LRM-322

SV-EC

14.1 Page 130

Change the last sentence of the Section 14.1 to:

Although SystemVerilog is not limited to simulation, the semantics of the language are defined for event directed simulation, and other uses of the hardware description language are abstracted from this base definition.

Done

# LRM-323

SV-EC

14.3 Page 131

Change the 5th paragraph after numbered time-slot list, to read as shown:

The sampling time of sampled data for property expressions is controlled in the clock domain block. The new #1step sampling delay provides the ability to sample data immediately before entering the current time slot, and is preferred over other equivalent constructs because it allows the 1step time delay to be parameterized. This #1step construct is a conceptual mechanism that provides a method for defining when sampling takes place, and does not require that an event be created in this previous time slot. Conceptually, this #1step sampling is identical to taking the data samples in the preponed region of the current time slot.

Done

# LRM-324

SV-EC

15.3 Page 138

Add the following paragraph at the end of this section.

Skews are declarative constructs, thus, they are semantically very different from the syntactically similar procedural delay statement.  In particular, a #0 skew, does not suspend any process nor does it execute or sample values in the Inactive region.

Done

# LRM-325

SV-EC

15.4 Page 138

Modify the 2nd paragraph as shown below:

However, hierarchical expressions are not limited to simple names or signals in other scopes. They can be used to declare slices and concatenations (or combinations thereof) of signals in other scopes or in the current scope.

Done

# LRM-326

SV-EC

15.10 Page 142

Modify the two examples at the top of the page as shown below:

## 5;	   // wait 5 cycles (clocking events) using the default clocking

## j + 1;	   // wait j+1 cycles (clocking events) using the default clocking